# Battery Management System (BMS) PoC - Backend Architecture Documentation

Tech Stack: FastAPI + Raspberry Pi + STM32 + Firebase RTDB + Firestore + React Dashboard

## 1. Objective

Build a proof-of-concept battery management platform where:

• Users configure battery profiles from a web dashboard.

• FastAPI distributes commands to Raspberry Pi over WebSocket.

• Raspberry Pi forwards MOSFET control instructions to STM32.

• STM32 controls MOSFET PWM/current profiles.

• Telemetry is streamed back in real time.

• FastAPI writes live data to Firebase RTDB.

• Historical packets are stored in Firestore.

• React dashboard visualizes real-time and historical analytics.

## 2. Final Communication Architecture

Browser  ←→  FastAPI     REST API      (config, profiles, analytics)

Browser  ←→  Firebase RTDB Firebase SDK  (real-time dashboard updates)

Pi       ←→  FastAPI     WebSocket       (commands + telemetry)

FastAPI  →   Firebase RTDB               (latest state/live stream)

FastAPI  →   Firestore                   (historical storage)

Reasoning:

• WebSocket provides bidirectional low-latency communication.

• Same socket handles both telemetry and control commands.

• RTDB provides instant UI synchronization.

• Firestore is optimized for querying historical data.

• FastAPI acts as orchestration layer and validation gateway.

## 3. Complete Backend Workflow

STEP 1 — User enters battery configuration

The React frontend collects:

• Battery chemistry

• Cell count

• Capacity

• Drone type (Agri / Surveillance / FPV)

• Discharge profile

STEP 2 — Frontend sends configuration to FastAPI

POST /api/start-profile

FastAPI:

• Validates payload

• Loads corresponding discharge dataset

• Creates execution session

• Pushes command to Raspberry Pi via WebSocket

STEP 3 — Raspberry Pi receives command

Pi runs:

• WebSocket client

• GPIO/PWM controller

• STM32 serial/UART bridge

Pi converts dataset timestamps into PWM/current instructions.

STEP 4 — STM32 controls MOSFET

STM32:

• Generates PWM signals

• Controls MOSFET gate

• Modulates battery discharge current

STEP 5 — Telemetry returned

STM32 sends:

• Voltage

• Current

• Temperature

• Event flags

Pi forwards telemetry to FastAPI over same WebSocket.

STEP 6 — FastAPI processing

FastAPI:

• Calculates derived metrics

• Writes latest packet to RTDB

• Writes packet history to Firestore

• Broadcasts alerts if thresholds exceeded

STEP 7 — Dashboard update

React dashboard listens using Firebase onValue().

Charts update nearly instantly after RTDB writes.

## 4. Why WebSocket is the Best Choice

Advantages:

• Persistent connection

• No repeated HTTP handshake

• Very low latency

• Bidirectional communication

• Efficient telemetry streaming

• Efficient command delivery

Alternative approaches:

1. HTTP Polling

Problem:

• High latency

• High overhead

• Poor scalability

2. MQTT

Good for production IoT systems but:

• Additional broker complexity

• Overkill for PoC

3. WebRTC

Not suitable for telemetry/control systems.

Final Recommendation:

Use FastAPI WebSockets for PoC simplicity and performance.

## 5. MOSFET Control Flow

User selects a discharge profile.

Dataset example:

timestamp,current_target

0,2A

5,5A

10,8A

FastAPI sends dataset reference to Pi.

Pi executes:

• Reads profile timestamps

• Converts current target into PWM duty cycle

• Sends PWM values to STM32

STM32:

• Drives MOSFET gate

• Current changes according to PWM duty cycle

Result:

Battery drains according to simulated drone load pattern.

## 6. Raspberry Pi Responsibilities

The Raspberry Pi acts as edge controller.

Responsibilities:

• Maintain WebSocket connection

• Receive commands

• Stream telemetry

• Translate profiles into STM32 instructions

• Cache commands during disconnects

• Handle UART/SPI communication

• Local fail-safe logic

## 7. FastAPI Responsibilities

Core Backend Services:

• Authentication

• Session management

• Dataset selection

• WebSocket manager

• Metric calculation

• Firebase integration

• Alert engine

• Historical analytics APIs

Recommended modules:

/api

/ws

/services

/models

/firebase

/analytics

/workers

## 8. Firebase Usage Strategy

Firebase RTDB:

Purpose:

• Live dashboard

• Current session state

• KPI widgets

• Real-time charts

Firestore:

Purpose:

• Historical telemetry

• Analytics

• Past sessions

• Long-term storage

Reasoning:

RTDB is optimized for real-time synchronization.

Firestore is optimized for structured historical querying.

## 9. Derived Metrics

SOC (State of Charge)

Estimated using:

• Coulomb counting

• Voltage estimation

SOH (State of Health)

Calculated from:

• Internal resistance growth

• Capacity degradation

RUL (Remaining Useful Life)

Predicted from:

• Cycle behavior

• Temperature stress

• Degradation patterns

Internal Resistance (IR)

Derived from:

IR = ΔV / ΔI

FastAPI analytics service computes these continuously.

## 10. Suggested Firebase Data Structure

RTDB:

live_sessions/

session_id/

latest_packet/

current_soc/

current_soh/

alerts/

Firestore:

telemetry_history/

session_id/

packet_documents/

profiles/

datasets/

analytics/

## 11. Real-Time Dashboard Features

Frontend Dashboard:

• Live voltage graph

• Current graph

• Temperature graph

• SOC gauge

• SOH gauge

• RUL estimate

• Active alerts

• Session logs

• Historical playback

Recommended React libraries:

• Recharts

• React Query

• Firebase SDK

• Zustand or Redux

## 12. Performance Expectations

Expected latency:

STM32 → Pi : 5–20ms

Pi → FastAPI WebSocket : 20–40ms

FastAPI → RTDB : 20–60ms

RTDB → React Dashboard : 50–100ms

Estimated total:

Under 200ms end-to-end.

This is effectively real-time for battery telemetry systems.

## 13. Security Recommendations

For PoC:

• API key authentication

• Firebase rules

• Device registration

For production:

• JWT authentication

• TLS everywhere

• Device certificates

• Rate limiting

• Signed firmware updates

## 14. Final Recommended Architecture

FINAL STACK

Frontend:

• ReactJS

• Firebase SDK

• Recharts

Backend:

• FastAPI

• WebSocket Manager

• Firebase Admin SDK

Edge:

• Raspberry Pi

• Python WebSocket client

• UART bridge

Embedded:

• STM32

• PWM + ADC

• MOSFET drivers

Database:

• Firebase RTDB

• Firestore

This architecture is optimal for a PoC because:

• Low complexity

• Near real-time behavior

• Easy visualization

• Simple deployment

• Good scalability path

## System Architecture Diagram

Attached architecture diagram reference: battery_mgmt_architecture.svg

## 15. Conclusion

The selected architecture using FastAPI WebSockets, Raspberry Pi edge control, STM32 MOSFET management, Firebase RTDB for live updates, and Firestore for history provides a strong low-latency PoC platform for intelligent battery management systems.